# Experiment 2

# Network Topology creation using Scilab

Name: Muthu Venkatesh M

Reg no:24BCE1389

PART 1:

CODE:

clc;

clear;

close;

n = 25;

edges = [

1 2; 1 3; 1 4; 1 5; 1 6; 1 7; 1 8;

9 10; 10 11; 11 12; 12 13; 13 14; 14 15; 15 16; 16 9;

17 18; 17 19; 18 20; 18 21; 19 22; 19 23; 21 24; 21 25;

8 9;

16 17

];

coords = rand(2, n);

scf(1);

clf();

a = gca();

a.data_bounds = [0 0; 1.2 1.2];

a.box = "on";

for k = 1:size(edges,1)

i = edges(k,1);

j = edges(k,2);

xpoly([coords(1,i) coords(1,j)], [coords(2,i) coords(2,j)]);

e = gce();

if (i <= 8 & j <= 8) then

e.foreground = 2;        // Star edges

elseif (i >= 9 & i <= 16) then

e.foreground = 3;        // Ring edges

elseif (i >= 17) then

e.foreground = 4;        // Tree edges

else

e.foreground = 5;        // Hybrid links

end

xm = (coords(1,i) + coords(1,j)) / 2;

ym = (coords(2,i) + coords(2,j)) / 2;

xstring(xm, ym, string(k));

end

for i = 1:n

xfarc(coords(1,i)-0.015, coords(2,i)-0.015, 0.03, 0.03, 0, 360*64);

c = gce();

if i <= 8 then

c.foreground = 2;        // Star nodes

elseif i <= 16 then

c.foreground = 3;        // Ring nodes

else

c.foreground = 4;        // Tree nodes

end

xstring(coords(1,i)+0.02, coords(2,i)+0.02, string(i));

end

deg = zeros(1,n);

for k = 1:size(edges,1)

deg(edges(k,1)) = deg(edges(k,1)) + 1;

deg(edges(k,2)) = deg(edges(k,2)) + 1;

end

disp("Node    Number of Edges");

for i = 1:n

disp([i deg(i)]);

end

[maxEdges, maxNode] = max(deg);

disp("Node with maximum edges:");

disp(maxNode);

disp("Maximum edges:");

disp(maxEdges);

disp("Total number of nodes:");

disp(n);

disp("Total number of edges:");

disp(size(edges,1));

OUTPUT:

PART B:

A large university campus network is designed using a hybrid topology to efficiently manage communication among different departments.

- The Administration Block uses a Star topology where one central server connects multiple offices for fast decision-making.

- The Academic Blocks are connected using a Ring topology to ensure equal data access and continuous communication between departments.

- The Hostel and Residential Area follows a Tree topology to support hierarchical data distribution across multiple buildings and floors.

- To ensure seamless communication across the campus, the Star, Ring, and Tree topologies are interconnected, forming a hybrid network with 25 nodes.

Each node represents a network device such as a router, switch, or workstation, and each edge represents a communication link.

### Questions:

- Identify the type of network topology used in the campus network and justify why a hybrid topology is suitable for this scenario.

- Determine the total number of nodes and edges in the network.

- Calculate the degree of each node and identify the node with the maximum number of connections.
Explain why this node plays a critical role in the network.